# Kinematic Analysis of a Four-Bar Mechanism: Merging Mechanism Animation, Velocity Analysis, and Acceleration Analysis

## Title Page

Title: Kinematic Analysis of a Four-Bar Mechanism: Merging Mechanism Animation, Velocity Analysis, and Acceleration Analysis
Author: (Your Name)
Institute: (Your Institute / University Name)
Department: Mechanical Engineering
Course: Theory of Machines
Date: (Date of Submission)

## Abstract

The four-bar mechanism is a fundamental planar linkage used extensively in mechanical systems to convert motion and transmit force. This report presents a concise and formal kinematic study that integrates position, velocity and acceleration analyses together with mechanism animation implemented in MATLAB. Starting from vector-loop kinematic relations, the study computes link orientations for a continuous range of crank inputs, differentiates these results to obtain velocities and accelerations, and visualizes the motion through animated plots. The combined numerical and visual approach clarifies critical kinematic behaviour such as velocity and acceleration peaks near toggle positions and provides evidence for engineering decisions like actuator selection and structural reinforcement. Sample MATLAB scripts and project files are referenced to enable reproduction and further extension. The numerical example uses the link set a = 50 mm, b = 70 mm, c = 90 mm, d = 100 mm with an input crank speed of omega_1 = 20 rad/s. Figures include representative plots of angular velocity and angular acceleration vs crank angle, plus schematic diagrams and animation instructions.

Keywords: Four-bar linkage, kinematic analysis, MATLAB animation, velocity analysis, acceleration analysis

## 1. Introduction

Mechanisms are assemblies of rigid bodies designed to produce constrained motion. The four-bar mechanism — a four-link closed planar chain with revolute joints — is among the most important basic linkages in mechanical design. Its simple topology enables complex coupler motions and diverse applications where precise motion profiles are required.

Objective. This report aims to present a professional, compact kinematic analysis of a four-bar mechanism, combining: (i) vector-loop position analysis, (ii) analytic velocity and acceleration evaluation, and (iii) animated visualization in MATLAB to aid interpretation and design decisions.

### 1.1 Engineering relevance

Four-bar linkages are used in automotive linkages, engine mechanisms, robotic arms, presses, and packaging equipment. Understanding kinematic behaviour influences sizing of actuators, dynamic balancing and fatigue life estimations.

## 2. Geometry and Configuration of the Four-Bar Mechanism

A standard four-bar mechanism consists of four rigid links connected by revolute joints. Denote link lengths as: a (crank), b (coupler), c (follower), d (ground).

Joint positions are taken in a plane with the ground link of length d fixed horizontally between two ground pivots.

### 2.1 Grashof criterion

Let s be the shortest and l the longest link, with the remaining two links p, q. The Grashof condition for full rotation of the shortest link is:

s + l <= p + q.

If satisfied, the linkage is a Grashof mechanism; classification (crank–rocker, double-crank, double-rocker) depends on which link is shortest and its adjacency to the ground.

## 3. Position Analysis and Animation (MATLAB emphasis)

### 3.1 Vector-loop relation

Using vector components, the loop closure can be written as:

r1 + r2 - r3 - r4 = 0

where r1 is the crank vector of magnitude a and angle theta, r2 the coupler b at angle phi, r3 the follower c at angle gamma, and r4 the ground vector d (angle zero in the chosen frame).

Writing components yields two scalar equations in the unknown output angles phi and gamma for each input crank angle theta. These are solved numerically (for example, using fsolve or closed-form relations when possible).

### 3.2 MATLAB algorithm (steps only — not full code)

- Define link lengths a, b, c, d, coupler offset e and range of input crank angles theta (e.g., 0–360 degrees with suitable step).

- For each theta: compute intermediate distance using cosine rule, determine coupler and follower angles using inverse trigonometric relations (vector-loop solution).

- Compute coordinates of joint points from link angles.

- Plot links as line segments and update the plot frame-by-frame to animate continuous motion.

- Save frame images or write a video file if required (VideoWriter in MATLAB).

### 3.3 How animation links to kinematics

Animation frames correspond to discrete values of input crank angle. By computing kinematic variables at each frame, the motion becomes a visual playback of the mechanism, enabling immediate inspection of coupler paths, interferences and extreme positions.

Files provided: Full interactive MATLAB Live Scripts collected during development are attached as project files at the following local paths (open these in MATLAB):

- /mnt/data/four_bar_animation.mlx

- /mnt/data/fourbar_position_velocity_acceleration.mlx

(These files contain runnable code, animation setup, and plotting instructions.)

## 4. Velocity Analysis (Analytical approach)

### 4.1 Fundamentals

Differentiate the scalar vector-loop equations with respect to time to obtain linear relations among angular velocities. If theta is crank angle and theta_dot = omega_1 its angular velocity, then:

-a*sin(theta)theta_dot - bsin(phi)phi_dot + csin(gamma)gamma_dot = 0
acos(theta)theta_dot + bcos(phi)phi_dot - ccos(gamma)*gamma_dot = 0

These two linear equations in unknowns phi_dot and gamma_dot can be arranged in matrix form and solved at each crank angle to obtain angular velocities.

### 4.2 Derived linear velocities

Once angular velocities are known, linear velocity of any joint point (for radius r from its pivot) is: v = omega * r.

Vector components can be computed as needed to obtain joint linear velocities and velocity polygons.

### 4.3 MATLAB steps for velocity

- After position solution, compute phi and gamma arrays.

- For each frame, assemble coefficient matrix and solve for phi_dot, gamma_dot.

- Store angular velocities and compute linear velocities for joint points.

- Plot omega vs theta and joint speed vs theta.

## 5. Acceleration Analysis

### 5.1 Components of link acceleration

For rotational motion, acceleration has radial (centripetal) and tangential components:

a_r = omega^2 * r
a_t = alpha * r

where alpha is angular acceleration.

### 5.2 Analytical acceleration relations

Differentiate the velocity equations to produce linear equations in angular accelerations phi_ddot, gamma_ddot. Arrange these into matrix form to solve for unknown angular accelerations given known velocities and input angular acceleration theta_ddot. Often theta_ddot = 0 for constant speed input.

### 5.3 MATLAB steps for acceleration

- Compute derivatives numerically (e.g., using central differences) or solve the analytic linear system at each frame.

- Compute radial and tangential accelerations for link points and display them as vectors on animation frames or as separate plots alpha vs theta.

## 6. Integration: Animation with Velocity & Acceleration

### 6.1 Mapping data onto animation

For each animation frame (corresponding to a theta_k), overlay:

- Velocity vectors at joints (scaled arrows) showing instantaneous linear velocity direction and magnitude.

- Acceleration vectors (radial + tangential decomposition).

- Color-coding or marker sizes can indicate magnitude (e.g., hotter color for higher magnitude).

### 6.2 MATLAB visualization tips

- Use quiver to plot velocity/acceleration vectors.

- Use patch for shaded coupler paths and plot for coupler curve.

- Use VideoWriter to export the animation to an mp4 for presentation.

## 7. Results and Discussion (Numerical Example)

### 7.1 Example parameters

Use:
a = 50 mm, b = 70 mm, c = 90 mm, d = 100 mm, e = 33 mm, alpha = 40 degrees
Input speed: omega_1 = 20 rad/s (constant)

The supplied MATLAB live scripts (/mnt/data/four_bar_animation.mlx and /mnt/data/fourbar_position_velocity_acceleration.mlx) compute position, angular velocities and accelerations across theta = 50 degrees to 410 degrees. Sample graphs of angular velocity and angular acceleration (generated during development) are presented in the figures attached to this report.

### 7.2 Representative plots

- Figure 1: Angular velocity of the coupler/follower vs crank angle (sample).

- Figure 2: Angular acceleration vs crank angle (sample).

Both figures show characteristic peaks near geometric critical positions (toggle positions) where link geometry produces rapid angular variation. These peaks are expected and indicate positions where actuator torque and structural demand are high.

## 8. Practical Interpretation

- Peaks in angular velocity or acceleration correspond to geometric configurations that magnify small input displacements into large output motions (near singular/toggle positions).

- For design: select motors and reducers to supply peak torque and account for transient accelerations.

- For fatigue and safety: reinforce links or redesign link proportions to reduce extremes if operational cycle spends significant time near peaks.

## 9. Conclusions

A professional kinematic analysis integrates position, velocity and acceleration computations with animation to reveal where a mechanism will experience demanding motions. MATLAB provides an effective platform to compute, visualize and export results for presentation and design actions. The included example demonstrates the expected qualitative patterns and provides a template for further dynamic or optimization studies.

## Appendix A — Key MATLAB Algorithm (Representative snippets)

Below are compact, representative MATLAB code snippets illustrating the numerical approach. These are not full scripts; full live scripts are provided as attached mlx files.

Position solution (per angle)

% Given a,b,c,d,theta (rad)

f = sqrt(a^2 + d^2 - 2*a*d*cos(theta));

G = atan2(a*sin(theta), d - a*cos(theta));

Bita = acos((f.^2 + b^2 - c^2)./(2*b.*f)) - G; % coupler angle

X = a*cos(theta) + e*cos(ar + Bita);

Y = a*sin(theta) + e*sin(ar + Bita);

Velocity solution (matrix form)

% Known: theta, phi, gamma, theta_dot

A = [-b*sin(phi), c*sin(gamma);

b*cos(phi), -c*cos(gamma)];

rhs = theta_dot * [a*sin(theta); -a*cos(theta)];

omega = A \ rhs; % [phi_dot; gamma_dot]

Acceleration solution (symbolic layout)

% Using known velocities phi_dot,gamma_dot and input accel theta_ddot

% Build coefficient matrix same as velocity and assemble RHS including

% terms with squared velocities; then solve for [phi_ddot; gamma_ddot]

phi_ddot = A \ rhs_accel;

Animation outline

figure('Color','w');

for k = 1:length(theta_array)

% compute joint coordinates

% update plot handles for crank,coupler,follower

% quiver for velocity and acceleration arrows

drawnow;

writeVideo(videoObj, getframe(gcf));

end

close(videoObj);

Note: Full runnable Live Script files are attached in the project folder paths above.

## References

- R. L. Norton, Design of Machinery, McGraw-Hill Education.

- J. J. Uicker, G. R. Pennock, J. E. Shigley, Theory of Machines and Mechanisms, Oxford University Press.

- S. S. Rattan, Theory of Machines, Tata McGraw-Hill Education.

- A. Ghosh, A. K. Mallik, Theory of Mechanisms and Machines, East-West Press.

## Acknowledgements

Project files and animations were prepared with MATLAB. Live scripts used for animation and plots are included in the attached mlx files for reproducibility.